Annual Drinking Water Quality Report for 2025

Town of Campbell

8529 Main Street

Campbell, NY 14821

Water Districts #1 NY5030104 & #4

INTRODUCTION

To comply with State regulations, the Town of Campbell, will be annually issuing a report describing the quality of your drinking water.  The purpose of this report is to raise your understanding of drinking water and awareness of the need to protect our drinking water sources.  Last year, your tap water met all State drinking water health standards.  We are proud to report that our system did not violate a maximum contaminant level or any other water quality standard.  This report provides an overview of last year’s water quality.  Included are details about where your water comes from, what it contains, and how it compares to State standards.

If you have any questions about this report or concerning your drinking water, please contact Thomas Austin, Water System Operator, at 607-769-5123. We want you to be informed about your drinking water.  If you want to learn more, please attend any of our regularly scheduled Town Board meetings. The meetings are held on the second Monday every month at 6:00p.m. located at the Town Hall at 8529 Main Street Campbell, NY 14821.

WHERE DOES OUR WATER COME FROM?

In general, the sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activities.  Contaminants that may be present in source water include: microbial contaminants; inorganic contaminants; pesticides and herbicides; organic chemical contaminants; and radioactive contaminants.  In order to ensure that tap water is safe to drink, the State and the EPA prescribe regulations which limit the amount of certain contaminants in water provided by public water systems.  The State Health Department’s and the FDA’s regulations establish limits for contaminants in bottled water which must provide the same protection for public health.

Our water system, at the time of this report, serves approximately 630 residents (excluding commercial business) through 261 service connections. Our water supply comes from two drilled wells, with depths of approximately 75 feet and 81 feet. The two wells are located at 8330 County Route 333. Both of the wells are treated with sodium hypochlorite and treated for iron, manganese, arsenic and methane prior to entering the distribution system. At the time of this report, a source water assessment was not available from the NYS Department of Health. For any questions regarding this assessment, please contact the Hornell District Office of the NYS Department of Health at 607-324-8371.

ARE THERE CONTAMINANTS IN OUR DRINKING WATER?

As the State regulations require, we routinely test your drinking water for numerous contaminants. These contaminants include: total coliform, inorganic compounds, nitrate, nitrite, lead and copper, volatile organic compounds, total trihalomethanes, halo acetic acids, radiological and synthetic organic compounds. The table presented below depicts which compounds were detected in your drinking water.  The State allows us to test for some contaminants less than once per year because our levels were at or below the detection levels.  Some of our data, though representative, are more than one year old.

It should be noted that all drinking water, including bottled drinking water, may be reasonably expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the EPA’s Safe Drinking Water Hotline (800-426-4791) or the Hornell District Office of the NYS Department of Health at 607-324-8371.

| Table of Detected Contaminants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contaminant | Violation Yes/No | Date of Sample | Level Detected (Avg/Max) (Range) | Unit Measure-ment | MCLG | Regulatory Limit (MCL, TT or AL) | Likely Source of Contamination |
| Inorganic Contaminates |  |  |  |  |  |  |  |
| Lead *1 | No | 9/16/2024 | 90% = 1.1 Range: ND-1.1 | ug/l | 0 | AL = 15 | Corrosion of household plumbing systems; Erosion of natural deposits. |
| Copper *1 | No | 9/16/2024 | 90% = 0.45 Range: 0.08-0.66 | mg/l | 1.3 | AL = 1.3 | Corrosion of household plumbing systems; Erosion of natural deposits; leaching from wood preservatives. |
| Arsenic | Yes | Quarterly 2025 | Ave: 4.545 Range: 3.4-4.9 | ug/l | N/A | 10 | Erosion of natural deposits; Runoff from orchards; Runoff from glass electronics production wastes. |
| Sodium** | No | 8/13/25 | 43.7 | mg/l | N/A | 0 | Naturally occurring; road salt; water softeners; animal waste. |
| Flouride | No | 11/4/2025 | 0.2 mg/L | mg/l | 2.2 | N/A | Some people who drink water containing fluoride in excess of the MCL over many years could get bone disease, including pain and tenderness of the bones. Children may get mottled teeth. |
| Barium | No | 11/4/2025 | 0.981 mg/L | mg/l | 2.0 | 2 | Some people who drink water containing barium in excess of the MCL over many years could experience an increase in their blood pressure |
| Nitrate | No | 8/13/25 | 0.0838 mg/L | Mg/l | 10 | 0.5 | Runoff from fertilizer use; Leaching from septic tanks, sewage; Erosion of natural deposits. |
| Chlorine | No | Monthly 2025 | Ave: 1.35 Range: 0.8-1.8 | mg/l | 4.0 | N/A | By-product of drinking water chlorination needed to kill harmful organisms. TTHMs are formed when source water contains large amounts of organic matter. |
| Organic Contaminates |  |  |  |  |  |  |  |
| Total Trihalomethanes | No | 8/21/2023 | 51 | ug/l | N/A | 80 | By-product of drinking water chlorination needed to kill harmful organisms. TTHMs are formed when source water contains large amounts of organic matter. |
| Total Trihalomethanes @ Entry Point Chloroform Bromodichloromethane Dibromochloromethane Bromoform | No No No No | 11/4/2025 114/2025 11/4/2025 11/4/2025 | 0.5 ug/L 1.5 ug/L 3.5 ug/L 5.1 ug/L | ug/L ug/L ug/L ug/L | NA NA NA NA | NA NA NA NA | By-product of drinking water chlorination needed to kill harmful organisms. TTHMs are formed when source water contains large amounts of organic matter. |
| Haloacetic Acids | No | 8/21/2023 | 11 | ug/l | N/A | 60 | By-product of drinking water chlorination. |

According to State regulations, the Town of Campbell Water District #1 routinely monitors your drinking water for various contaminates. Your water is tested for nitrates, bromomethane, chlorthane, and chloromethane. Additionally, your water is tested for coliform bacteria once per month. The contaminants detected in your drinking water are included in the Table of Detected Contaminants.

*1 –Lead and Copper Results: The level presented is the 100th percentile of the 5 sites tested. A percentile is a value on a scale of 100 that indicates the percentage of measurements that are equal to or below it. The action level for copper was not exceeded at any of the sites tested.

** - Water containing more than 20 mg/L of sodium should not be used for drinking water by people on severely restricted sodium diets. Water containing more than 270 mg/L of sodium should not be used for drinking by people on moderately restricted diets.

Definitions:

Maximum Contaminant Level  (MCL): The highest level of a contaminant that is allowed in drinking water.  MCLs are set as close to the MCLGs as feasible.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health.  MCLGs allow for a margin of safety.

Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water.  There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health.  MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contamination.

Action Level  (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water.

Non-Detects (ND): Laboratory analysis indicates that the constituent is not present.

Nephelometric Turbidity Unit (NTU): A measure of the clarity of water. Turbidity in excess of 5 NTU is just noticeable to the average person.

Milligrams per liter (mg/l): Corresponds to one part of liquid in one million parts of liquid (parts per million - ppm).

Micrograms per liter (ug/l): Corresponds to one part of liquid in one billion parts of liquid (parts per billion - ppb).

Nanograms per liter (ng/l): Corresponds to one part of liquid to one trillion parts of liquid (parts per trillion - ppt).

Picograms per liter (pg/l): Corresponds to one part per of liquid to one quadrillion parts of liquid (parts per quadrillion – ppq).

Picocuries per liter (pCi/L): A measure of the radioactivity in water.

Millirems per year (mrem/yr): A measure of radiation absorbed by the body.

Million Fibers per Liter (MFL): A measure of the presence of asbestos fibers that are longer than 10 micrometers.

WHAT DOES THIS INFORMATION MEAN?

As you can see by the table, our system had no violations.  We have learned through our testing that some contaminants have been detected; however, these contaminants were detected below current federal drinking water requirements.  Although arsenic was detected below the MCL, it was detected at 6.4 which is greater than one-half of the MCL. Therefore, we are required to present the following information on arsenic in drinking water:

“NYS and EPA have promulgated a drinking water arsenic standard of 10 parts per billion.  While your drinking water meets the standard for arsenic, it does contain low levels of arsenic.  The standard balances the current understanding of arsenic’s possible health effects against the costs of removing arsenic from drinking water.  EPA continues to research the health effect of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.”

Lead can cause serious health effects in people of all ages, especially pregnant people, infants (both formula-fed and breastfed), and young children. Lead in drinking water is primarily from materials and parts used in service lines and in home plumbing. The Town of Campbell is responsible for providing high quality drinking water and removing lead pipes but cannot control the variety of materials used in the plumbing in your home. Because lead levels may vary over time, lead exposure is possible even when your tap sampling results do not detect lead at one point in time. You can help protect yourself and your family by identifying and removing lead materials within your home plumbing and taking steps to reduce your family's risk. Using a filter, certified by an American National Standards Institute accredited certifier to reduce lead, is effective in reducing lead exposures. Follow the instructions provided with the filter to ensure the filter is used properly. Use only cold water for drinking, cooking, and making baby formula. Boiling water does not remove lead from water. Before using tap water for drinking, cooking, or making baby formula, flush your pipes for several minutes. You can do this by running your tap, taking a shower, doing laundry or a load of dishes. If you have a lead service line or galvanized requiring replacement service line, you may need to flush your pipes for a longer period. If you are concerned about lead in your water and wish to have your water tested, contact Tom Austin at 607-769-5123 for Water Districts 1 & 4 Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available at https://www.epa.gov/safewater/lead.

IS OUR WATER SYSTEM MEETING OTHER RULES THAT GOVERN OPERATIONS?

We are required to monitor your drinking water for specific contaminants on a regular basis.  Results of regular monitoring are an indicator of whether or not your drinking water meets health standards During 2025, our system was in compliance with applicable State drinking water operating, monitoring and reporting requirements with no violations.

INFORMATION ON LEAD SERVICE LINE INVENTORY

A Lead Service Line (LSL) is defined as any portion of pipe that is made of lead which connects the water main to the building inlet. An LSL may be owned by the water system, owned by the property owner, or both. The inventory includes both potable and non-potable SLs within a system. In accordance with the federal Lead and Copper Rule Revisions (LCRR) our system has prepared a lead service line inventory and have made it publicly accessible by (include instructions on how to access the inventory (including inventories consisting only of a statement declaring that the distribution system has no lead, galvanized requiring replacement, or lead status unknown service lines)) and/or visiting our website at: https://www.campbellny.com/water-district.html

DO I NEED TO TAKE SPECIAL PRECAUTIONS?

Some people may be more vulnerable to disease causing microorganisms or pathogens in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice from their health care provider about their drinking water.  EPA/CDC guidelines on appropriate means to lessen the risk of infection by Cryptosporidium, Giardia and other microbial pathogens are available from the Safe Drinking Water Hotline (800-426-4791).

CLOSING

Thank you for allowing us to continue to provide your family with quality drinking water this year. In order to maintain a safe and dependable water supply we sometimes need to make improvements that will benefit all of our customers. The costs of these improvements may be reflected in the rate structure. Rate adjustments may be necessary in order to address these improvements.  We ask that all our customers help us protect our water sources, which are the heart of our community.  Please call our office if you have questions.

Sincerely,

Tom Austin, Water System Operator